# Building Security Plan

This document intends to clearly layout security recommendations for the building. It should serve as a guide to all physical security mechanisms that should be in place. The following recommendations are based off the level of security necessary for the buildings assets and should not be compromised upon. All security personnel should review and sign this document to be aware of the security mechanisms in place.

# Perimeter

Fencing:

- Perimeter fencing must surround the entire premises

- Fencing should be 10-12 ft in height

- The fences should be made of welded mesh or steel palisade

- Steel fence posts in a concrete foundation

- Outriggers on top of the fence, angled outward. This could be adorned with barbed wire.

- Fibre Optic IDS should be mounted to the fence.

- The fencing should have a stand off distance of at least 100 feet from the building.

Access Point:

- A gated entryway should be implemented to allow authorized personnel in and out of the premises. The gate should be seamlessly integrated into the fencing.

- The gate should be constructed out of steel mesh that meet anti-ram standards

- A guard booth should be constructed and operated by a security guard 24/7

  - Those approaching the premises will be stopped and verified before proceeding. Cameras to identify license plate numbers will be in place. Visitors will need a badge/ID to be checked. Once verified, the guard will manually allow the gates to be open, with only enough time for the single entry. The guard may issue temporary access for authorized visitors. The guard will have access to an alarm lever in case of emergency.

- Speed bumps should be placed leading up to the gate

Additional Permitter Defenses:

- A CCTV system should be installed around the premises to cover the entire perimeter. Cameras should be a both HD cameras and thermal imaging. Cameras should be mounted on steel poles inside the fence.

- The surrounding permitter should be landscaped to minimize blind spots to provide an unobstructed view.

- Signage should be erected to notify entrants that the permitter is authorized access only & that the premises are monitored by CCTV.

- Trained security guards should do regular patrols of the permitter. This should include an audit of the defenses to ensure there has been no tampering and everything is working properly.

- Proper lighting should be installed to ensure clear visibility around the premises.

# Grounds & External Building

Grounds:

- The property surrounding the building should be landscaped to be clearly visible.

- Cameras are to be installed to cover all area within the fences

- Lighting should cover the area for clear visibility.

- Guards should regularly patrol the grounds.

Building Exterior:

- The building should feature walls that are resistant to forced entry

- Blast resistant windows should be installed. These should be treated to provide minimal visibility from the exterior.

- CCTV should provide a clear view of the entire building, especially entry points.

- External Doors should be composed of steel with a solid mineral core. Hinges should be heavy-duty and tamper proof. Multi-point Locking systems should be installed.

- A multi-factor authentication system should be implemented for entry. A PIV card reader and cards should be used as well as a keypad with a PIN inputted on the reader.

# Building Interior

Entryway:

- The entryway should be manned by trained guards. The will monitor entrants, issue temporary access, and ensure the safety of the entryway. They will

- Metal detectors & x-ray scanners may be installed to ensure prohibited items are not brought into the building.

Doors:

- Unrestricted areas should use laminate doors with mineral cores. They will use PIV cards for entry. They will use bolts and strike plate locks.

- Restricted areas will use steel doors with a mineral core. PIV and a pin code system will be used to access. Locks will be bolt and stroke plates.

Surveillance:

CCTV cameras will be installed to cover all entry points, corridors, and sensitive areas. All areas monitored should be properly lit and have no blind spots. Motion sensors will be in place for after hours.

The Server Room:

- The sever room should surrounded in fire-rated flooring, ceilings, walls and doors. There should be no plenum between the walls and ceiling.

- Access should be limited to authorized IT and security personnel only. It will implement both PIV card and pin access.

- Monitoring systems for temperature, humidity, smoke and water should be in place.

- Inert gas fire extinguishers should be in place in case of a fire.

- Servers should be behind locked cabinets with keys only held by authorized personnel.

# Security Guards

- Guards should be properly trained, know the layout of the property and do routine exercises. Training should cover emergency response plans, and lockdown procedures

- Guards should patrol the interior and exterior regularly

- Guards will monitor camera footage regularly

- Guards will be uniformed and easily identified

- Guards will be equipped with access methods, communication devices, etc.

# Technology

CCTV:

- CCTV footage will be received at a centralized VMS that allows access to live footage and recordings.

- CCTV should be equipped with detection software for license plates, movement and faces.

- Cameras should have 4K and thermal imaging

- Video feeds should be encrypted and transmitted over a dedicated network.

Alarms & Monitoring:

- Pulleys for fires and emergencies will be installed in convenient locations across the building.

- A centralized Security Operations Centre will receive all alarm and sensor data. Guards will use this to monitor systems and alerts, and dispatch alerts and response teams.

- If the IDS sensor is triggered, footage from the cameras will be pulled and alerts will be administered.

2025-12-08